Hi Alex,

Thanks for confirming your availability. Our call is scheduled as follows:

Date/Time: Oct 5, 2026 11:00am-11:30am (GMT-07:00) Pacific Time (US & Canada)
Interviewers: Samantha Buenrostro

Preferred Skills And Experience

- Master’s degree in a technical or engineering discipline

- Experience with physical troubleshooting of fiber

- Deep understanding of datacenter networks including but not limited to:

  - Connectivity (Ethernet, IPv4, IPv6 and MPLS)

  - Routing (BGP, OSPF, VXLAN)

  - Switching and routing

  - InfiniBand/NVLink

  - NVIDIA GPU deployment stacks (Blackwell/Rubin)

- Experience implementing network security functions

- Cisco, Juniper, Arista or other industry certification

- Ability to work independently and in a team, take initiative, and communicate effectively

- Ability to obtain and maintain a Top Secret or Top Secret SCI clearance

- Speeds and lane structures

- What a SerDes lane is

- Every high-speed Ethernet port is built from SerDes lanes (serializer/deserializer). A SerDes is the circuit on the switch ASIC or NIC that turns parallel data into a single high-speed serial stream, and each lane is one of those streams. Port speed is just lanes multiplied by per-lane rate, so the same switch chip can expose its bandwidth as a few very fast ports or many slower ones.

- For example, a 51.2 Tbps switch ASIC (like Broadcom Tomahawk 5 or NVIDIA Spectrum-4) has 512 lanes at 100G each. Grouped 8 lanes per port, that's 64 × 800G ports; grouped 4 per port, 128 × 400G; individually, 512 × 100G. The newest 102.4 Tbps chips move to 200G lanes, enabling 64 × 1.6T ports.

- NRZ versus PAM4

- How each lane encodes bits matters because it drives error rates and FEC requirements. NRZ (non-return-to-zero) uses two signal levels, so one bit per symbol; it was used through 25G per lane. PAM4 (pulse amplitude modulation, four levels) carries two bits per symbol, doubling throughput at the same signaling rate, and is used for 50G, 100G, and 200G lanes.

- The cost of PAM4 is that four levels are packed closer together, so the signal is much more sensitive to noise. That's why PAM4 links essentially always need strong forward error correction, and why some errors on a PAM4 link are normal and expected.

- How the common speeds are built

| Port speed | Typical lane structure | Encoding | Common form factor |
| --- | --- | --- | --- |
| 25G | 1 × 25G | NRZ | SFP28 |
| 100G (older) | 4 × 25G | NRZ | QSFP28 |
| 100G (newer) | 2 × 50G or 1 × 100G | PAM4 | QSFP28/QSFP56, single-lane optics |
| 400G | 8 × 50G or 4 × 100G | PAM4 | QSFP-DD, OSFP, QSFP112 |
| 800G | 8 × 100G (today), 4 × 200G (emerging) | PAM4 | OSFP, QSFP-DD800 |
| 1.6T | 8 × 200G | PAM4 | OSFP (OSFP-XD in future) |

- Two speeds that look identical can be incompatible. A 100G-SR4 optic (4 × 25G NRZ over multimode) won't talk to a 100G-DR optic (1 × 100G PAM4 over single-mode), even though both are "100G." When a link won't come up between two ports that both claim the same speed, lane structure and encoding mismatch is one of the first things to check.

- Electrical lanes versus optical lanes

- The electrical lanes from the ASIC don't always match the optical lanes in the transceiver. A 400G-DR4 optic uses four optical lanes at 100G each over four fiber pairs (MPO-12 connector). A 400G-FR4 optic multiplexes four wavelengths (CWDM) onto a single fiber pair with a duplex LC connector. When the host side runs 8 × 50G electrical but the optic runs 4 × 100G optical, a gearbox inside the module converts between them. Gearboxes add power, heat, and latency, which is one reason the industry moved toward matching 100G electrical and optical lanes.

- Form factors

- QSFP-DD (double density) and OSFP (octal small form factor) are both 8-lane form factors. OSFP is slightly larger, handles more power and heat, and is what NVIDIA uses heavily in its AI networking gear. NVIDIA's switches commonly use twin-port OSFP modules: one 800G cage containing two independent 400G ports, each with its own MPO connector. That design lets one switch port feed two separate 400G GPU NICs.

- FEC

- FEC (forward error correction) adds redundant data so the receiver can fix bit errors without retransmission. The two codes you'll hear most are RS(528,514), often called "KR4 FEC," used with 25G NRZ lanes, and RS(544,514), called "KP4 FEC," required for PAM4 lanes. Both ends of a link must agree on FEC mode, or the link either stays down or comes up with massive errors. FEC mismatch is a classic cause of breakout links failing.

- FEC also gives you useful diagnostics. Switches report pre-FEC BER (errors before correction) and post-FEC or uncorrectable errors. A healthy PAM4 link shows some corrected errors and essentially zero uncorrectable ones. A rising pre-FEC error rate is an early warning that an optic or fiber is degrading, often days before the link actually fails. In large GPU clusters, monitoring this is how you catch bad links before they slow down training jobs.

- Breakout cabling

- What breakout means

- Breakout splits one high-speed port into several lower-speed ports by giving each subset of lanes its own connection. A 400G port with four 100G lanes can become four independent 100G ports; an 800G port with eight 100G lanes can become two 400G ports or eight 100G ports.

- Common physical forms include optical breakout, where a 400G-DR4 optic with an MPO-12 connector feeds an MPO-to-4×LC breakout cable, with each LC pair landing on a separate 100G-DR optic at the far end. Copper breakout DACs (direct attach cables) split one 400G or 800G connector into multiple smaller connectors for short in-rack runs. Twin-port OSFP cabling, as above, splits an 800G cage into two 400G links.

- Why breakout is used

- In AI clusters, breakout is how switch radix gets matched to NIC speed. If your spine switch has 800G ports but your GPU NICs are 400G ConnectX-7 cards, breakout lets each switch port serve two NICs instead of wasting half its bandwidth. It's also used to reduce switch count: one 64-port 800G switch broken out to 400G behaves like a 128-port 400G switch.

- What goes wrong

- Breakout is a frequent source of bring-up problems, and knowing the failure modes is what distinguishes real experience.

- Configuration comes first. The switch port must be explicitly configured for breakout mode (the exact command varies by vendor), and after that the interface naming changes: Ethernet1/1 might become Ethernet1/1/1 through 1/1/4. Configuration written for the unbroken port no longer applies.

- Speed, lane count, and FEC must match on both sides. If the switch side presents 4 × 100G PAM4 and the far end expects 100G as 4 × 25G NRZ, the link won't come up. Similarly, both ends must run KP4 FEC.

- ASIC port-group limits can bite. Many switch chips group ports into blocks sharing hardware resources, so breaking out one port may disable an adjacent port, or only certain port numbers support breakout. Always check the platform's port-mapping documentation.

- Auto-negotiation and link training matter for copper. DAC cables typically rely on auto-negotiation and link training to tune the signal; optics usually don't. A DAC that won't come up often needs auto-negotiation explicitly enabled or disabled to match the other end.

- Cable mapping and polarity are the physical issues. With MPO breakout cables, a polarity error (Type A versus Type B) or a mislabeled leg means a lane lands on the wrong far-end port. At scale, careful labeling and cabling verification tools are essential.

- MTU and jumbo frames

- Basic definitions

- MTU (maximum transmission unit) is the largest packet an interface will send without fragmenting. Standard Ethernet MTU is 1500 bytes of payload. Jumbo frames raise this, usually to 9000 bytes on hosts and 9216 bytes on switches (the switch value is set a bit higher to leave headroom for encapsulation and tags).

- Why GPU and storage traffic uses jumbo frames

- Every packet carries fixed overhead: headers, plus per-packet processing in the NIC, kernel, or switch. Larger packets mean fewer packets for the same data, so less overhead, lower CPU and NIC load, and higher effective throughput. For GPU training (moving huge gradients between nodes) and storage traffic (NFS, NVMe over Fabrics, parallel file systems like Lustre or WEKA), jumbo frames meaningfully improve performance.

- The vendor-counting trap

- A common real-world gotcha is that vendors don't define MTU the same way. Some count only the IP packet (Layer 3 MTU); others include the 14-byte Ethernet header (and sometimes the VLAN tag) in their interface MTU. So "MTU 9000" on one platform may not equal "MTU 9000" on another, and mixed-vendor environments can end up mismatched by a few bytes. Knowing exactly what each platform counts is a mark of real experience.

- Encapsulation overhead

- Tunnels eat into MTU. VXLAN adds about 50 bytes with an IPv4 outer header (outer Ethernet, IP, UDP, and VXLAN headers) and about 70 bytes with IPv6. If hosts send 9000-byte packets into a VXLAN fabric, the underlay must support at least 9050 bytes, which is one reason switches are set to 9216. MPLS labels add 4 bytes each, and 802.1Q VLAN tags add 4 bytes.

- RoCE has its own MTU

- This is a detail that impresses interviewers. RDMA over Converged Ethernet uses its own MTU, inherited from InfiniBand, with fixed values of 256, 512, 1024, 2048, or 4096 bytes. The NIC picks the largest RDMA MTU that fits within the Ethernet MTU after headers. So even on a 9000-byte Ethernet network, RoCE traffic usually runs with a 4096-byte RDMA MTU. If the Ethernet MTU is accidentally left at 1500, RDMA silently falls back to 1024, which can significantly cut throughput without any obvious error. On Linux, ibv_devinfo shows the active MTU for each RDMA port.

- Why mismatches are "silent"

- MTU problems are notorious because small packets work fine, so basic connectivity tests pass. Ping succeeds, SSH connects, and the problem only appears when large packets are sent, as transfers that hang, stall, or crawl.

- The

- Encapsulation overhead

- Tunnels eat into MTU. VXLAN adds about 50 bytes with an IPv4 outer header (outer Ethernet, IP, UDP, and VXLAN headers) and about 70 bytes with IPv6. If hosts send 9000-byte packets into a VXLAN fabric, the underlay must support at least 9050 bytes, which is one reason switches are set to 9216. MPLS labels add 4 bytes each, and 802.1Q VLAN tags add 4 bytes.

RoCE (pronounced "rocky") stands for RDMA over Converged Ethernet. It's a way of running RDMA, a very fast, low-overhead method of moving data between computers, over ordinary Ethernet networks. In AI clusters, it's the main alternative to InfiniBand for connecting GPUs across servers.

- RoCE has its own MTU

- This is a detail that impresses interviewers. RDMA over Converged Ethernet uses its own MTU, inherited from InfiniBand, with fixed values of 256, 512, 1024, 2048, or 4096 bytes. The NIC picks the largest RDMA MTU that fits within the Ethernet MTU after headers. So even on a 9000-byte Ethernet network, RoCE traffic usually runs with a 4096-byte RDMA MTU. If the Ethernet MTU is accidentally left at 1500, RDMA silently falls back to 1024, which can significantly cut throughput without any obvious error. On Linux, ibv_devinfo shows the active MTU for each RDMA port.

#### PFC (Priority Flow Control)

PFC (IEEE 802.1Qbb) lets a switch tell its upstream neighbor "pause sending traffic in this class" when its buffer for that class is filling up. Unlike old Ethernet pause frames, which stopped all traffic on a link, PFC works per priority class (there are eight), so RoCE traffic can be paused while other traffic keeps flowing. Traffic is typically classified using DSCP markings in the IP header, with RoCE assigned to its own lossless class.

PFC prevents drops, but it has known dangers. Head-of-line blocking and congestion spreading happen because a pause affects all flows in that class on the link, not just the flow causing congestion, so congestion can ripple backward through the fabric and slow unrelated traffic. A PFC storm occurs when a malfunctioning NIC or host sends pause frames continuously and freezes traffic across large parts of the network; switches use a PFC watchdog to detect and break this. PFC deadlock occurs when pauses form a circular dependency between switches and traffic stops entirely. It's rare in well-designed Clos fabrics but possible with routing loops or certain failure conditions.

Because of these risks, PFC is treated as a safety net, not the main congestion tool.

#### ECN and DCQCN

The main tool is end-to-end congestion control, so senders slow down before buffers fill. ECN (Explicit Congestion Notification) is the first piece: when a switch queue grows past a configured threshold, the switch marks packets with a congestion flag instead of dropping them. When the receiving NIC sees marked packets, it sends a Congestion Notification Packet (CNP) back to the sender. DCQCN is the rate-control algorithm built into the NICs: when the sender receives CNPs, it reduces its transmission rate, then gradually increases it again once congestion clears.

The design goal is that ECN and DCQCN handle congestion most of the time, and PFC only kicks in as a last line of defense. Much of the real engineering work is tuning ECN thresholds and buffer settings so the two work together: if ECN marks too late, PFC fires constantly; if it marks too early, throughput suffers. A common convention in NVIDIA-based deployments is to put RoCE data traffic in one priority class and CNPs in a separate high-priority class, so congestion signals themselves never get stuck behind congestion.

Routing is the heart of data center network design, and it's where interviews tend to go deepest. Here's each protocol, how it's used in modern fabrics, and how it applies to AI clusters specifically.

### BGP

#### The fundamentals

BGP (Border Gateway Protocol) is a path-vector protocol. Each route carries a list of the autonomous systems (ASNs) it has passed through, along with other attributes, and routers choose paths based on policy rather than a single metric. BGP runs over TCP port 179, so peering depends on a working TCP connection, which is a key point for troubleshooting.

There are two flavors. eBGP runs between different ASNs; routers rewrite the next hop to themselves by default, and a route containing your own ASN in its path is rejected as a loop. iBGP runs within the same ASN; it doesn't change the next hop by default, and routes learned from one iBGP peer aren't passed to another iBGP peer. That rule exists to prevent loops, but it means iBGP needs either a full mesh of sessions or route reflectors to distribute routes.

#### Session states

A BGP session moves through six states, and knowing where a session is stuck tells you what's wrong.

| State | Meaning | Stuck here usually means |
| --- | --- | --- |
| Idle | Not attempting to connect | Neighbor misconfigured, administratively down, or no route to peer |
| Connect | Attempting TCP connection | TCP in progress |
| Active | TCP connection failed, retrying | No IP reachability, ACL or firewall blocking port 179, wrong peer IP, or source address mismatch |
| OpenSent | TCP up, OPEN message sent | Waiting for peer's OPEN |
| OpenConfirm | OPEN received, waiting for keepalive | Parameter mismatch: wrong ASN, authentication failure, incompatible capabilities |
| Established | Session up, exchanging routes | Working. If routes are missing, look at policy |

A classic interview point: despite the name, "Active" is a bad state. It means BGP is actively trying and failing to establish TCP.

#### Best-path selection

When BGP has multiple paths to a prefix, it compares attributes in a fixed order. The common sequence (exact details vary slightly by vendor) is:

- Highest weight (Cisco-specific, local to the router)

- Highest local preference (shared within the ASN; used to pick exit points)

- Locally originated routes preferred

- Shortest AS path

- Lowest origin type (IGP over EGP over incomplete)

- Lowest MED (multi-exit discriminator; a hint to neighbors about which entry point to use)

- eBGP over iBGP

- Lowest IGP metric to the next hop

- Tiebreakers: oldest path, lowest router ID, lowest neighbor address

You don't need to recite every step in an interview, but knowing the top several, and that local preference controls outbound traffic while AS-path prepending and MED influence inbound traffic, is expected.

#### BGP in the data center

Large fabrics typically run eBGP everywhere, an approach documented in RFC 7938 and used widely by hyperscalers. The reasons are simplicity (one protocol for the whole network instead of an IGP plus BGP), scalability (BGP doesn't flood link-state information to every router, so failures stay local), strong policy control, and predictable failure behavior.

ASN design is a key decision. The common pattern is that all spines share one ASN, and each leaf (or leaf pair) gets its own ASN. Private ASNs are used: 64512–65534 in the 2-byte range, or 4200000000–4294967294 in the 4-byte range, which is preferred at scale because it provides millions of numbers.

Why do spines share an ASN? It prevents path hunting. If spines had unique ASNs, then when a leaf lost a route, other routers might briefly try longer paths through other spines and leaves (leaf → spine → leaf → spine → leaf) before converging. With a shared spine ASN, eBGP loop prevention automatically rejects any path that passes through two spines, so only valid two-hop paths exist. The same mechanism prevents "valley" routing, where traffic goes down to a leaf and back up again.

If leaves share an ASN instead (some designs reuse one ASN for all leaves to save numbers), they'd reject each other's routes as loops, so you'd need allowas-in to accept them. That's an important detail if you see it in a config.

#### ECMP

Equal-cost multipath is what makes a spine-leaf fabric work: a leaf with four spines should spread traffic across all four. Two settings matter. maximum-paths must be raised, because BGP installs only one best path by default. Multipath relax (bgp bestpath as-path multipath-relax on Cisco and Arista, multipath multiple-as on Juniper) is needed because standard BGP only load-balances across paths with identical AS paths. When equal-length paths come through neighbors with different ASNs, multipath relax lets BGP treat them as equal.

#### Fast failure detection

BGP's default timers are slow (often a 60-second keepalive and 180-second hold time, depending on vendor), which is far too slow for a data center. Fabrics use **BFD (Bidirectional Forwarding Detection)**, a lightweight protocol that detects link or peer failure in milliseconds (for example, 300 ms intervals with a multiplier of 3, detecting failure in under a second) and tells BGP to tear down the session immediately. Many fabrics also rely on the fact that a physical link going down instantly drops the BGP session on directly connected interfaces.

#### BGP unnumbered

In large fabrics, assigning an IPv4 address to every point-to-point link becomes a big administrative burden. BGP unnumbered peers over IPv6 link-local addresses that interfaces generate automatically, and carries IPv4 routes with IPv6 next hops (RFC 8950, formerly RFC 5549). You configure peering by interface name instead of IP address. It's common in SONiC and Cumulus-style environments and very popular in AI clusters because it makes configuration nearly identical across thousands of switches.

#### Route policy tools

Policy is where BGP expertise really shows. Prefix lists match specific routes or ranges. Route maps (or routing policies on Juniper) match routes and set attributes, which is the main policy tool. Communities are tags attached to routes; they come in standard, extended, and large forms, and they let you classify routes ("from rack 12," "GPU cluster," "do not export") and apply policy based on the tag rather than listing prefixes. AS-path prepending makes a path look longer to discourage its use. Local preference and MED steer traffic, as above. Maximum-prefix limits shut down or warn on a session that suddenly sends far more routes than expected, a key safety control against route leaks.

#### Example leaf configuration

Here's a simplified leaf configuration in Arista EOS style to show how these pieces fit together:

router bgp 4200001012

router-id 10.0.0.12

maximum-paths 64

bgp bestpath as-path multipath-relax

neighbor SPINES peer group

neighbor SPINES remote-as 4200000001

neighbor SPINES bfd

neighbor SPINES maximum-routes 12000

neighbor 10.1.12.0 peer group SPINES

neighbor 10.1.12.2 peer group SPINES

neighbor 10.1.12.4 peer group SPINES

neighbor 10.1.12.6 peer group SPINES

!

address-family ipv4

neighbor SPINES activate

network 10.0.0.12/32

network 172.16.12.0/24

This leaf has its own ASN, peers with four spines that share one ASN, enables BFD and ECMP across up to 64 paths, caps the number of routes it will accept, and advertises its loopback and rack subnet.

#### Troubleshooting BGP

A useful systematic approach starts by checking the session state, which tells you whether the problem is reachability (Idle or Active) or parameters (OpenSent or OpenConfirm). If the session is down, verify IP reachability to the peer, check that nothing is blocking TCP 179, and confirm ASN, authentication, and peer addresses match on both sides. If the session is established but routes are missing, check whether the routes are being advertised at all, whether inbound or outbound policy is filtering them, whether the next hop is reachable, and whether the route is being rejected as an AS-path loop. If routes are present but traffic isn't load-balancing, check maximum-paths and multipath relax.

Useful commands include show ip bgp summary (session states and prefix counts), show ip bgp neighbors <ip> (detailed session information and errors), show ip bgp <prefix> (all paths and which one won), and show ip bgp neighbors <ip> received-routes or advertised-routes to see what's actually being exchanged.

### OSPF

#### The fundamentals

OSPF (Open Shortest Path First) is a link-state protocol. Every router learns the full topology of its area by exchanging LSAs (link-state advertisements), builds an identical database, and runs Dijkstra's SPF algorithm to compute shortest paths using interface cost. OSPF runs directly over IP as protocol 89, not over TCP or UDP. OSPFv2 is for IPv4; OSPFv3 supports IPv6.

#### Neighbor states

OSPF adjacencies progress through a series of states, and like BGP, where a neighbor is stuck tells you what's wrong.

| State | Meaning | Stuck here usually means |
| --- | --- | --- |
| Down | No hellos received | No connectivity, interface not in OSPF, or passive interface |
| Init | Hello received, but this router isn't listed in it | One-way communication, often an ACL or multicast issue |
| 2-Way | Bidirectional communication | Normal for non-DR routers on broadcast segments |
| ExStart / Exchange | Negotiating and exchanging database descriptions | MTU mismatch (the classic cause) |
| Loading | Requesting missing LSAs | Database exchange problems |
| Full | Fully synchronized | Working |

For neighbors to form at all, they must agree on area ID, hello and dead timers (defaults are 10 and 40 seconds on broadcast networks), subnet and mask, authentication, and area type (such as stub flags). MTU mismatch doesn't prevent neighbors from forming, but it stops them reaching Full, which is why it shows up as ExStart or Exchange.

#### Network types and DR election

On broadcast networks like Ethernet, OSPF elects a designated router (DR) and backup to reduce the number of adjacencies. On point-to-point fabric links between two switches, this is unnecessary overhead and slows convergence, so data center designs configure fabric links as point-to-point network type to skip DR election entirely.

#### Areas and LSA types

OSPF scales using areas. Area 0 is the backbone, and all other areas must connect to it. An ABR (area border router) connects areas, and an ASBR (autonomous system boundary router) brings in external routes.

The common LSA types are Type 1 (router LSA; each router describes its own links), Type 2 (network LSA; generated by the DR for a broadcast segment), Type 3 (summary LSA; ABRs advertising routes between areas), Type 4 (ASBR summary; tells other areas how to reach an ASBR), Type 5 (external routes from an ASBR), and Type 7 (external routes inside an NSSA, a not-so-stubby area). Stub and NSSA areas filter some of these types to reduce database size.

#### OSPF in the data center

OSPF's main role in modern fabrics is as the underlay for an EVPN-VXLAN network. It provides reachability between switch loopbacks, while BGP EVPN carries the overlay information. IS-IS, another link-state protocol, is often used in the same role and is popular with some large operators.

Why did hyperscale fabrics largely move from link-state protocols to BGP? Because every router in an area must hold the full topology and recompute SPF when something changes, a single flapping link triggers flooding and recalculation across the whole area. At thousands of switches, that becomes a scaling and stability problem. BGP keeps failures more local and gives finer policy control. OSPF still works well for small and mid-sized fabrics and is simpler to configure for an underlay.

#### Troubleshooting OSPF

Check neighbor state first (show ip ospf neighbor). If neighbors don't appear, verify the interface is in OSPF and not passive, and check area, timers, subnet, and authentication. If stuck in ExStart or Exchange, check MTU on both sides. If neighbors are Full but routes are missing, examine the database (show ip ospf database), area types, and any filtering or summarization at ABRs.

### VXLAN and EVPN

#### What VXLAN is

Start with the important clarification: VXLAN isn't a routing protocol. It's an encapsulation (a tunneling method) that carries Layer 2 Ethernet frames inside UDP packets across a Layer 3 network. The routing protocol that makes VXLAN work at scale is usually BGP EVPN, which serves as its control plane.

#### Why it exists

A pure Layer 3 spine-leaf fabric is stable and scalable, but some workloads still need Layer 2 connectivity across racks: VM mobility, legacy applications, clustering software, or tenants who want their own private segments. VXLAN provides Layer 2 services on top of a Layer 3 network, keeping the fabric's stability while offering flexibility. It also solves the VLAN limit: VLANs offer only 4,094 IDs, while VXLAN's 24-bit VNI (VXLAN Network Identifier)allows about 16 million segments, which matters for multi-tenant environments.

#### How encapsulation works

A VTEP (VXLAN tunnel endpoint), usually the leaf switch, takes the original Ethernet frame and wraps it in an 8-byte VXLAN header containing the VNI, a UDP header (destination port 4789), an outer IP header addressed from the source VTEP's loopback to the destination VTEP's loopback, and an outer Ethernet header. The spine switches route the packet like any other IP packet and never need to know about the VXLAN segments inside.

Two practical details matter here. The outer UDP source port is derived from a hash of the inner packet, which gives the fabric entropy for ECMP so different inner flows spread across different paths. And encapsulation adds about 50 bytes of overhead (more with IPv6), which is why fabric MTU must be raised.

#### Handling broadcast, unknown unicast, and multicast

Layer 2 networks need to flood certain traffic (broadcasts like ARP, frames to unknown destinations, and multicast), together called BUM traffic. VXLAN handles this two ways. Multicast underlay maps each VNI to a multicast group in the underlay network; it's efficient but requires running multicast routing in the fabric, which adds complexity. Ingress replication (head-end replication) has the source VTEP send a separate unicast copy to every other VTEP in the segment. It's simpler and more common today, especially with EVPN.

#### EVPN as the control plane

Early VXLAN used flood-and-learn: VTEPs learned remote MAC addresses by watching flooded traffic, much like a traditional switch. This doesn't scale well and generates lots of unnecessary flooding.

EVPN (Ethernet VPN) solves this by using BGP (specifically the L2VPN EVPN address family of multiprotocol BGP) to advertise MAC addresses, IP addresses, and VTEP membership proactively. Each VTEP learns where every host lives through BGP, the way routers learn routes, instead of discovering it through flooding.

#### EVPN route types

| Type | Name | Purpose |
| --- | --- | --- |
| 1 | Ethernet auto-discovery | Multihoming: fast convergence and aliasing |
| 2 | MAC/IP advertisement | Advertises a host's MAC address and optionally its IP |
| 3 | Inclusive multicast (IMET) | Advertises VTEP membership in a VNI; builds the flood list for BUM traffic |
| 4 | Ethernet segment | Multihoming: discovers peers on a shared segment and elects the designated forwarder |
| 5 | IP prefix | Advertises IP subnets, used for routing between segments and to external networks |

Types 2, 3, and 5 come up most often in day-to-day operation. Types 1 and 4 relate to EVPN multihoming, covered in the Ethernet section earlier.

#### Route distinguishers and route targets

Two concepts borrowed from MPLS VPNs control how EVPN routes are handled. A route distinguisher (RD) is prepended to routes so that identical addresses in different tenants stay unique in BGP. A route target (RT) is an extended community that controls which VTEPs import which routes: a VTEP imports routes whose RT matches its configured import targets.

Route-target mismatch is one of the most common EVPN problems. The BGP session is up, routes are being advertised, but the receiving VTEP silently ignores them because the RTs don't match.

#### L2VNIs, L3VNIs, and routing between segments

An L2VNI represents a Layer 2 segment, roughly equivalent to a VLAN extended across the fabric. An L3VNIrepresents a tenant's routing instance (VRF) and is used to carry routed traffic between segments.

There are two models for routing between segments. In asymmetric IRB, the ingress VTEP does both routing and bridging and sends the packet directly into the destination's L2VNI. It's simpler but requires every VTEP to have every VNI configured, which doesn't scale. In symmetric IRB, both ingress and egress VTEPs route: the packet travels across the fabric in the tenant's L3VNI and is routed again at the destination. VTEPs only need the VNIs for their locally connected hosts, so this is the scalable model and the common choice today.

#### Distributed anycast gateway

In an EVPN fabric, every leaf acts as the default gateway for its local hosts, using the same gateway IP address and MAC address on every leaf. Hosts always route through their directly attached leaf, traffic doesn't hairpin through a central router, and a host can move between racks without changing its gateway configuration.

#### ARP suppression

Because EVPN Type 2 routes already tell each VTEP the MAC-to-IP mappings for remote hosts, the local leaf can answer ARP requests on behalf of remote hosts instead of flooding them across the fabric. This significantly reduces BUM traffic.

#### Underlay and overlay design choices

There are two common EVPN fabric designs. IGP underlay with iBGP overlay uses OSPF or IS-IS for loopback reachability and iBGP EVPN for the overlay, with spines acting as route reflectors. This is common in Cisco-style designs. eBGP underlay with eBGP overlay uses eBGP for both, often on the same sessions or with separate ones. This is common in Arista, SONiC, and hyperscale-style designs. One important detail here is that spines must be configured to leave the next hop unchanged for EVPN routes, so traffic tunnels directly between leaf VTEPs rather than to the spine.

#### Troubleshooting VXLAN and EVPN

A layered approach works well. First, verify the underlay: can each VTEP reach the other VTEPs' loopback addresses, and is MTU large enough for encapsulated traffic? Then check the EVPN BGP sessions, making sure they're established with the L2VPN EVPN address family active. Next, check route exchange: are Type 2, 3, and 5 routes present, and do route targets match? Then check VTEP peers (on Cisco, show nve peers; on Arista, show vxlan vtep) and confirm the VNI-to-VLAN mappings and VRF configuration are correct. Finally, check the MAC and ARP tables to see whether remote hosts have been learned through EVPN.

### How this applies to AI clusters

Different parts of an AI cluster use routing differently, and describing this well in an interview shows you understand the whole picture.

The GPU backend (compute) fabric is usually pure Layer 3 with eBGP, often using BGP unnumbered or /31 point-to-point links, with no VXLAN overlay. The goal is maximum simplicity, performance, and predictability; encapsulation overhead and Layer 2 complexity add nothing here. Some multi-tenant GPU cloud providers do add overlays on the backend to isolate customers from each other, but dedicated training clusters generally keep it simple.

The frontend network, which connects GPU servers to users, schedulers, the internet, and the rest of the company, often uses EVPN-VXLAN, because it needs multi-tenancy, segmentation, flexible placement, and integration with other networks.

The storage network may be pure Layer 3 or EVPN-based depending on the storage system, often running RoCE for high-performance parallel file systems.

The out-of-band management network is typically simple, separate, and conservative, so it stays reachable when everything else breaks.

SONiC, an open-source network operating system originally developed at Microsoft, is common in large AI clusters and pairs naturally with eBGP and BGP unnumbered designs. Familiarity with it is a plus.

### Likely interview questions

Expect questions like: Why do data centers use eBGP instead of OSPF? Why do spines share an ASN? What is path hunting? What does multipath relax do? A BGP session is stuck in Active; what do you check? An OSPF neighbor is stuck in ExStart; what's wrong? Is VXLAN a routing protocol? What do EVPN Type 2, 3, and 5 routes do? What's the difference between symmetric and asymmetric IRB? EVPN sessions are up but hosts can't reach each other across the fabric; how do you troubleshoot? How would you design routing for a GPU backend fabric versus the frontend network?

"BGP is more scalable" is the claim. "Because OSPF floods topology and reruns SPF everywhere" is the mechanism. "So failures stay contained at thousands of switches" is the consequence. 

Because BGP passes routes along to its neighbors, the prefix doesn't only travel directly from spine to leaf. It also bounces further. S1 sends it to L2, and L2 passes it back up to S2. So S2 now has the direct path from L1 (AS path 65101) and a longer backup path through S1 and L2 (AS path 65102 65001 65101). BGP doesn't reject that longer path, because no ASN appears twice in it. These longer paths go "down" to a leaf and back "up," which is called valley routing, and they're never paths you'd want traffic to take.

Now put both spines in the same ASN, say 65000.

When S1 sends the prefix to L2 and L2 passes it up to S2, the AS path would be 65102 65000 65101. S2 sees its own ASN, 65000, in that path. eBGP's standard loop-prevention rule says: reject any route containing your own ASN. So S2 drops it immediately.

That means spines can only ever learn the prefix directly from the leaf that owns it. The longer leaf-spine-leaf-spine paths are never accepted in the first place, so when L1 fails there are no backup paths to hunt through. The withdrawal propagates once, and the fabric converges cleanly and quickly.

### In short

### Path hunting is BGP cycling through progressively longer, invalid backup paths after a route is withdrawn, delaying convergence. Spines share an ASN so that eBGP's built-in loop prevention automatically rejects any path crossing two spines. The only paths that exist are the valid ones, so there's nothing to hunt through when something fails.

The classic cause: MTU mismatch

This is the answer interviewers are looking for first. Every DBD packet includes the sending interface's MTU. Per the OSPF specification, if a router receives a DBD from a neighbor whose MTU is larger than its own interface MTU, it rejects the packet, because the neighbor might later send packets too big for it to handle.

So the exchange can't complete. The neighbors formed fine, because hellos are small and don't check MTU, but the database sync never finishes. Typically, the router with the larger MTU sits in ExStart retransmitting DBDs, while the one with the smaller MTU keeps rejecting them.

You'd confirm it by comparing MTU on both interfaces (show ip ospf interface or show interface). Debugging OSPF adjacencies on Cisco often shows a message saying the neighbor has a larger interface MTU.

The proper fix is to make the MTUs match. There's also a command, ip ospf mtu-ignore, that tells OSPF to skip the check. It works, but it's a workaround that hides the real problem: if the MTU actually differs, large LSA updates may later be dropped, causing subtler failures. A strong candidate mentions it exists and explains why you shouldn't rely on it.

### Other causes worth mentioning

Mentioning these after MTU shows depth. Duplicate router IDs can cause it, because if both routers have the same router ID, they can't settle who's master. Unicast reachability problems are another: hellos are multicast, but on some network types the DBD exchange uses unicast, so an ACL, firewall, or one-way issue can let hellos through while blocking DBDs. A device in the middle with a smaller MTU is subtle: the two routers' interfaces match, but a switch or link between them can't pass large packets, so the larger DBDs (and later LSA updates) get dropped. Testing with large do-not-fragment pings across the link reveals it.

### A strong interview answer

"ExStart is where the neighbors negotiate master and slave and begin exchanging database description packets. The most common reason for getting stuck there is an MTU mismatch: DBD packets carry the interface MTU, and a router rejects DBDs from a neighbor with a larger MTU, so the database sync never completes even though hellos work fine. I'd check the MTU on both interfaces and fix it so they match. There's an mtu-ignore option, but I'd avoid it since it hides the real problem. If MTUs match, I'd check for duplicate router IDs, anything blocking unicast OSPF packets, or a device in the path with a smaller MTU dropping larger packets."

That answer follows the same claim, mechanism, and consequence pattern, and it ends with a troubleshooting order, which interviewers like because it shows how you'd actually work the problem.

"Active" is one of the most misleading names in networking. It sounds healthy, but it means BGP is actively trying to establish a TCP connection to its neighbor and failing. The session never gets far enough to exchange BGP messages.

### Where Active fits

BGP moves through Idle, Connect, Active, OpenSent, OpenConfirm, and finally Established. In Connect, BGP is attempting the TCP three-way handshake on port 179. If that attempt fails, it moves to Active, where it keeps listening for and retrying the connection. A session sitting in Active, or bouncing between Active and Idle, means the TCP connection itself isn't completing.

That's the key insight: Active is almost always a transport problem, not a BGP parameter problem. Parameter mismatches like a wrong ASN usually show up later, after TCP is up.

### What to check, in order

1. Peer address configuration. Confirm the neighbor IP is correct on both sides, and that the remote router actually has a matching neighbor statement. If the remote has no config for you, it typically resets the TCP connection immediately.

2. Basic reachability. Ping the peer, and do it from the same source address BGP is using, for example ping 10.0.0.2 source loopback0. A plain ping can succeed while the BGP source fails.

3. Source address (update-source). This is a very common cause when peering between loopbacks. If you peer to the neighbor's loopback but don't set update-source loopback0, your router sources the TCP connection from the outgoing physical interface. The remote sees a connection from an address it doesn't recognize and rejects it. You also need a route to the peer's loopback, through an IGP or static route.

4. eBGP TTL (multihop). eBGP sends packets with a TTL of 1 by default, assuming the peer is directly connected. If you peer eBGP between loopbacks, packets need at least two hops, so you need ebgp-multihop 2 (or disable-connected-checkon some platforms). Otherwise the packets expire before reaching the peer.

5. Anything blocking TCP 179. ACLs, firewalls, or control-plane policing can block the connection. You can test the port directly with telnet <peer> 179 from the right source. A connection that's refused or times out points here.

6. MD5 or TCP-AO authentication mismatch. If one side has a password and the other doesn't (or they differ), the TCP segments get silently discarded, so the session stays in Active. Logs usually show an invalid or missing MD5 digest message, which makes this one quick to confirm.

7. Wrong VRF. If the neighbor is configured in one VRF but the peer is reachable in another, TCP never connects.

If the session briefly reaches OpenSent or OpenConfirm and then falls back to Active or Idle, the problem has shifted from transport to BGP parameters: wrong remote ASN, hold-time issues, or capability mismatches. Check the last reset reason and NOTIFICATION message with show ip bgp neighbors <ip>, which usually names the exact error.

### A strong interview answer

"Active means BGP is trying to open a TCP connection on port 179 and failing, so it's really a transport problem rather than a BGP parameter issue, despite the name. I'd first confirm the neighbor addresses match on both sides, then test reachability from the same source address BGP uses. If we're peering between loopbacks, I'd check update-source and, for eBGP, multihop or TTL settings. Then I'd check for ACLs, firewalls, or control-plane policing blocking TCP 179, and for an MD5 password mismatch, which silently drops the TCP packets. If the session gets to OpenSent and then drops, I'd look at the last NOTIFICATION, which usually points to an ASN or capability mismatch."

### The strongest signal in that answer is the opening line. Saying "Active is bad, and it's a TCP problem" right away tells an interviewer you've actually troubleshot BGP before.

1. Is VXLAN a routing protocol?

"No. VXLAN is an encapsulation, a data-plane technology. It wraps Layer 2 Ethernet frames inside UDP packets, destination port 4789, with an 8-byte header carrying a 24-bit VNI, so you can stretch Layer 2 segments across a routed Layer 3 fabric. VXLAN itself doesn't decide where anything goes. The underlay still needs a routing protocol, like eBGP or OSPF, to reach the VTEP loopbacks, and VXLAN needs a control plane to learn where hosts live. That's either flood-and-learn, which doesn't scale well, or more commonly BGP EVPN, which advertises MAC and IP information through BGP."

Why it works: It answers "no" immediately, then separates data plane, underlay, and control plane. That three-way distinction is exactly what the question is testing.

### 2. What do EVPN Type 2, 3, and 5 routes do?

"Type 2 is the MAC/IP advertisement. When a leaf learns a local host, it advertises the host's MAC address, optionally its IP, and the VNI it belongs to, so every other VTEP knows exactly which leaf that host sits behind. Because the IP is included, remote leaves can answer ARP locally, which is ARP suppression. Type 2 routes also carry a sequence number for MAC mobility, so the fabric handles hosts moving between racks.

Type 3 is the inclusive multicast route. Each VTEP advertises which VNIs it participates in, and other VTEPs use that to build their flood lists for BUM traffic with ingress replication. If Type 3 routes are missing, broadcasts like ARP never reach the remote VTEP, and hosts can't find each other at all.

Type 5 is the IP prefix route. It advertises whole subnets into a tenant VRF rather than individual hosts, used for external routes coming in from border leaves, routing between fabrics, and reaching silent hosts that haven't generated a Type 2 yet."

Why it works: Each route type gets its purpose plus a practical consequence. The note that missing Type 3 breaks ARP shows you've connected the theory to real failures.

### 3. Symmetric versus asymmetric IRB

"Both are ways of routing between subnets in an EVPN fabric. Picture Host A in VLAN 10 on Leaf 1 and Host B in VLAN 20 on Leaf 2.

With asymmetric IRB, Leaf 1 does all the work. It routes the packet from VLAN 10 into VLAN 20 locally, then encapsulates it in VLAN 20's VNI and sends it across. Leaf 2 just bridges it to Host B. On the way back, Leaf 2 routes and sends it in VLAN 10's VNI, so the two directions use different VNIs, which is why it's called asymmetric. The catch is that every leaf must have every VLAN and VNI configured, along with all the MAC and ARP state that comes with them, so it doesn't scale.

With symmetric IRB, both leaves route. Leaf 1 routes the packet into the tenant's L3VNI, which represents the VRF, sends it across the fabric, and Leaf 2 routes it again from the L3VNI into VLAN 20. Both directions use the same L3VNI. Leaves only need the VLANs for their own locally attached hosts, plus the L3VNI per VRF, and the leaves exchange router MAC addresses in their EVPN routes so they can rewrite headers. It costs an extra lookup, but it scales far better and supports Type 5 routes, so it's the standard choice in modern fabrics."

Why it works: The concrete two-host example makes an abstract topic easy to follow, and explaining why the name is "asymmetric" shows real understanding rather than memorization.

### 4. EVPN sessions are up, but hosts can't reach each other

"Since the BGP sessions are up, the control plane is at least partly working, so I'd go layer by layer.

First, the underlay. Can the VTEP loopbacks reach each other? I'd ping loopback to loopback, then send a large do-not-fragment ping to check that MTU is big enough to carry VXLAN overhead. MTU problems here are very common, because small packets like pings work while real traffic fails.

Second, VTEP discovery. Does each leaf see the other as a VTEP peer, with show nve peers on Cisco or show vxlan vtep on Arista? If not, check whether Type 3 routes are being received for that VNI.

Third, route exchange. Is there a Type 2 route for the remote host? If the route is in the BGP table but not installed, the usual suspect is a route-target mismatch: the route arrives but the VTEP doesn't import it. In an eBGP overlay, I'd also confirm the spines are leaving the next hop unchanged, so it still points at the remote leaf's VTEP rather than the spine.

Fourth, local configuration. Are the VLAN-to-VNI mappings consistent on both leaves? For routed traffic, is the VRF and L3VNI configured the same on both, and is the anycast gateway using the same IP and MAC everywhere?

Fifth, the host's own leaf. Is the host's MAC actually learned locally? If the access port is in the wrong VLAN, the leaf never learns the host and never advertises a Type 2 route in the first place.

I'd also check traffic in both directions, because one-way failures are common in VXLAN, and finally confirm the entries are programmed in hardware, not just in software."

Why it works: It's a clear, ordered method instead of a list of guesses, and it calls out the most common real causes, MTU and route-target mismatch, by name.

### 5. Routing design for a GPU backend fabric versus the frontend network

"The two networks have opposite priorities, so I'd design them very differently and keep them physically separate.

The backend fabric exists for one job: GPU-to-GPU traffic for collective operations like all-reduce, over RDMA. Everything is optimized for performance and predictability. I'd build a non-blocking, rail-optimized Clos, so each GPU's NIC connects to its own rail of leaf switches. Routing would be pure Layer 3 with eBGP, using BGP unnumbered or /31 point-to-point links, a unique ASN per leaf, and a shared ASN on the spines to avoid path hunting. I'd enable wide ECMP with multipath relax, and use fast failure detection. There'd be no VXLAN overlay, because encapsulation and Layer 2 complexity add nothing here. Policy stays minimal, mainly advertising host links or rack summaries. If it's RoCE, PFC and ECN settings must be consistent end to end, and adaptive routing helps with the large elephant flows that hash poorly. If the backend is InfiniBand instead, routing is handled by the subnet manager rather than BGP.

The frontend network connects GPU servers to users, schedulers, storage, the internet, and corporate networks, so it's optimized for flexibility and security. I'd use EVPN-VXLAN with symmetric IRB, VRFs to separate security zones such as production, management, and user access, distributed anycast gateways, and EVPN multihoming so servers are dual-attached. Border leaves would connect to the WAN, internet, and cloud, with strict route filtering, summarization, and firewalls at the zone boundaries.

Storage would get its own network or a dedicated, lossless class, and out-of-band management would stay completely separate, simple, and reachable even when everything else is down."

Why it works: It opens with the core principle (opposite priorities, so opposite designs), then backs each choice with a reason. Mentioning that InfiniBand doesn't use BGP at all shows awareness of the full landscape, which matters for this SpaceX role.