# Why Use VXLAN/EVPN?

## Executive Summary

VXLAN and EVPN are used together to build scalable, redundant, and multi-tenant data-center networks.

- VXLAN provides the data-plane transport by carrying Layer 2 Ethernet frames across a routed Layer 3 network.

- EVPN provides the control plane by using BGP to advertise MAC addresses, IP addresses, prefixes, VTEP locations, and multihoming information.

Together, they solve many of the scaling and operational limitations of traditional VLAN-based data-center designs.

The maximum 802.1Q VLAN ID space is:

- 4,096 total values: VLAN IDs 0–4095

- 4,094 usable VLANs: VLAN IDs 1–4094

- VLAN 0: Reserved for priority tagging

- VLAN 4095: Reserved

With VXLAN/EVPN, the overlay uses 24-bit VNIs, allowing up to approximately 16.7 million VNI values. However, the practical limit depends on the switch ASIC, NOS, and platform scale—not just the protocol.

## Problems with Traditional VLAN Networks

Traditional Layer 2 designs have several limitations:

- VLAN IDs provide only a limited number of logical segments.

- Large Layer 2 domains depend heavily on STP or MLAG.

- Extending VLANs across many racks becomes complex.

- Traffic may follow inefficient Layer 2 paths instead of using all available links.

- Multi-tenant isolation becomes difficult at scale.

- MAC learning is primarily data-plane-based.

- Endpoint movement can create reconvergence and stale-state problems.

- Layer 2 and Layer 3 services are often tightly coupled to the physical topology.

## What VXLAN Solves

VXLAN carries Layer 2 Ethernet frames across a routed IP underlay.

Original Ethernet frame

↓

VXLAN encapsulation

↓

UDP/IP packet across the routed underlay

↓

Decapsulation at the remote leaf

VXLAN provides:

- A much larger logical segment space through 24-bit VNIs.

- Layer 2 extension across a Layer 3 leaf-spine fabric.

- ECMP load sharing across the underlay.

- Reduced dependence on STP in the core.

- Separation between the physical underlay and tenant overlay.

- Consistent tenant networks across different leaf switches.

A VTEP, or VXLAN Tunnel Endpoint, performs VXLAN encapsulation and decapsulation. In most data-center designs, leaf switches act as VTEPs.

## What EVPN Solves

VXLAN by itself does not provide a complete control plane. Without EVPN, VTEPs may rely on flood-and-learn behavior or multicast-based discovery.

EVPN uses BGP to advertise:

- MAC addresses.

- MAC and IP bindings.

- VTEP locations.

- IP prefixes.

- Multihoming information.

- BUM-replication membership.

- MAC mobility events.

This provides a more controlled and visible way to distribute endpoint reachability information.

### Important EVPN Route Types

- Type 2: Advertises MAC and IP endpoint information.

- Type 3: Advertises BUM-replication membership.

- Type 4: Advertises Ethernet multihoming information.

- Type 5: Advertises IP prefixes across the EVPN fabric.

## What VXLAN and EVPN Solve Together

### 1. Network Scale

VLANs identify local Layer 2 segments. VNIs provide a much larger overlay namespace for tenants, networks, and services.

### 2. Layer 2 Connectivity Across a Routed Fabric

A host on Leaf 1 can communicate with a host in the same logical network on Leaf 2 without extending a physical Layer 2 domain through the spine layer.

### 3. Better Use of Network Links

The underlay is routed and can use ECMP. Traffic can use multiple spine paths instead of relying on a spanning-tree-controlled topology.

### 4. Multi-Tenancy

Each tenant can have:

- Its own VRF.

- Its own Layer 2 VNIs.

- Its own Layer 3 VNI.

- Overlapping IP address space.

- Controlled route import and export policies.

### 5. Active-Active Redundancy

EVPN multihoming allows servers, firewalls, and other devices to connect to multiple leaf switches and actively use both paths.

### 6. Endpoint Mobility

When a VM or workload moves to another leaf, EVPN advertises the new MAC/IP location and uses MAC mobility information to update the fabric.

### 7. Integrated Layer 2 and Layer 3 Services

The same fabric can support:

- Layer 2 tenant segments.

- Inter-VLAN routing.

- VRF isolation.

- External connectivity.

- Shared services.

- Firewall insertion.

- Internet access.

## Example: Same-Subnet Communication

Consider this topology:

Host A — Leaf 1/VTEP — Spine — Leaf 2/VTEP — Host B

The forwarding process is:

- Leaf 1 learns Host A's MAC address.

- EVPN advertises Host A's MAC/IP information using a Type 2 route.

- Leaf 2 learns that Host A is reachable through Leaf 1's VTEP address.

- Leaf 1 encapsulates Host A's Ethernet frame with VXLAN.

- The routed underlay delivers the packet to Leaf 2.

- Leaf 2 removes the VXLAN header and forwards the original Ethernet frame to Host B.

For communication between different subnets, EVPN Type 5 routes and Layer 3 VNIs can advertise tenant prefixes across the fabric.

## Technology Responsibilities

| Technology | Primary responsibility |
| --- | --- |
| VXLAN | Encapsulates and transports Layer 2 traffic across a Layer 3 network |
| EVPN | Uses BGP to advertise endpoint, prefix, and multihoming information |
| BGP underlay | Provides IP reachability between leafs, spines, and VTEPs |
| VRF and VNI | Provide tenant and routing-table isolation |
| Leaf switches | Connect hosts and provide VTEP and gateway functions |
| Spine switches | Provide routed transit and ECMP paths |

## Tradeoffs and Operational Considerations

VXLAN/EVPN is powerful, but it introduces additional complexity:

- BGP and EVPN must be understood.

- The underlay must support VXLAN overhead.

- MTU must be consistent end to end.

- Route targets and VNIs must be configured consistently.

- Underlay failures can affect overlay reachability.

- Troubleshooting requires checking multiple layers.

- Vendor implementations can differ.

- Monitoring must cover physical, underlay, overlay, and tenant state.

## When VXLAN/EVPN Is Appropriate

VXLAN/EVPN is most valuable when the environment requires:

- Large-scale data-center growth.

- Many tenants or isolated services.

- Layer 2 mobility across racks.

- Active-active redundancy.

- ECMP across a leaf-spine fabric.

- Consistent Layer 2 and Layer 3 services.

- Flexible workload placement.

- Automation and intent-based provisioning.

For a small or simple network, traditional VLANs and routing may be easier to operate. VXLAN/EVPN becomes valuable when scale, multi-tenancy, mobility, redundancy, and efficient use of the routed fabric are more important than minimizing architectural complexity.

## Key Interview Explanation

A concise interview answer is:

VXLAN solves the data-plane scaling problem by carrying Layer 2 segments across a routed IP fabric using VTEPs and VNIs. EVPN solves the control-plane problem by using BGP to advertise MACs, IPs, prefixes, and multihoming information. Together, they provide scalable Layer 2 and Layer 3 connectivity, multi-tenancy, ECMP, workload mobility, and active-active redundancy in modern data centers.